# Activity Hazard Analysis (AHA) — Asbestos Abatement

SAMPLE formatted to EM 385-1-1 (15 March 2024) and ENG Form 6206 (SEP 2024) conventions. Adapt every step, hazard and control to actual site conditions; requires competent-person preparation, SSHO review and Government acceptance before the definable feature of work begins (EM 385-1-1 2024, para 2-6). Contracts solicited before 15 March 2024 remain governed by the 2014 edition — verify which edition your contract cites.

| Activity/Work Task (DFOW): Asbestos Abatement | Date: | Project Location: | Contract Number: |
| --- | --- | --- | --- |
| Prepared By (Name/Title): | Reviewed By (Name/Title): | Notes: | Overall RAC: M |

# Job Steps, Hazards and Controls

| JOB STEPS | HAZARDS | CONTROLS (actions to eliminate or minimize hazards) | RESIDUAL RAC |
| --- | --- | --- | --- |
| Mobilization, survey review, and regulatory notification | Unidentified or mischaracterized ACM disturbed because survey data is incomplete or not keyed to the work areas | • Obtain and review the asbestos survey required by EM 385-1-1 para 17-8.b, performed by trained and licensed inspectors, before pricing or sequencing any disturbance work. • Describe each activity that will disturb ACM in the written Asbestos Management Plan — OSHA class of work, crew size, procedures, and ACM components keyed to the project drawings (para 6-7.d(1)). • Treat any suspect material not covered by the survey as presumed ACM; stop work and sample before disturbance (29 CFR 1926.1101(k)). • Label all ACM as an asbestos hazard not to be disturbed without proper protection; where ACM is inaccessible to label, create and post a site map identifying hazard locations (para 6-8.f(1)(c)). | L |
|  | Regulatory violation from missing NESHAP notification or uncoordinated site employers | • Submit the 40 CFR 61 Subpart M notification to the administering agency at least 10 working days before regulated renovation/demolition work begins. • Implement the Asbestos Management Plan method for notifying other employers at the work site of regulated areas and scheduled abatement (para 6-7.d(2)). • Verify the AHA with its identified RAC has been reviewed, accepted, and signed by the approval authority (para 2-6.c(1)) and the Asbestos Management Plan has been accepted before mobilizing abatement crews. | L |
| Regulated area setup: critical barriers, containment, and decontamination unit | Fiber release beyond the work area from incomplete containment or unsealed penetrations | • Establish the regulated area, containment type, and decontamination unit exactly as described in the accepted Asbestos Management Plan (para 6-7.d(3)); demarcate and sign the regulated area per 29 CFR 1926.1101(e). • Isolate and seal HVAC intakes, grilles, and duct penetrations serving the work area before disturbing any ACM; coordinate system shutdown and lockout with the facility. • Post environmental (perimeter) air monitoring to demonstrate the absence of fiber migration outside the regulated area (para 6-7.d(4)(c)). | L |
|  | Electrical shock from temporary power, wet methods, and negative air machines in a wet enclosure | • Feed all containment power tools, negative air machines, and lighting through GFCI protection and inspect cords before each shift. • Route cords overhead and off wet floors; de-energize and lock out circuits within the containment that will be wetted or cut. • Use low-voltage or double-insulated lighting inside shower and wash-down areas. | L |
|  | Falls from ladders and baker scaffolds while hanging poly on ceilings and critical barriers | • Select ladders and rolling scaffolds rated for the task; maintain three points of contact and do not stand on the top two ladder steps. • Keep floor poly taut and taped, and dry-mop slick surfaces before repositioning ladders. • Provide fall protection for any barrier work at 6 ft or more above lower levels per EM 385-1-1 Ch 21. | L |
| Pre-abatement verification: negative pressure, smoke test, and worker clearance | Containment failure not detected before disturbance work starts | • Verify negative pressure enclosures hold the design pressure differential (typically -0.02 in. w.g.) on a recording manometer and smoke-test barriers and airlocks before first disturbance. • Evaluate the ventilation (negative air) system to confirm it functions as designed (para 6-5.c) and inspect filtration units per the manufacturer's instructions. • Document the pre-abatement inspection in the daily log per the inspection frequency set in the Asbestos Management Plan. | L |
|  | Unqualified or medically unfit workers entering the regulated area | • Verify every entrant holds current asbestos training for their role and exposure potential per OSHA/EPA standards — typically EPA/AHERA worker and supervisor accreditation for Class I/II work (para 6-3.h(2)). • Confirm current medical surveillance and respirator medical clearance for each worker before donning (paras 6-7.d(9), 5-7.b(3)). • Confirm current fit tests for tight-fitting respirators and document them in the RPP records (para 5-7.b(4)). • Have every involved employee review and sign the AHA before starting work (para 2-6.d(2)). | L |
| ACM removal (Class I/II work) inside containment | Airborne asbestos fiber exposure above the 0.1 f/cc PEL during gross removal | • Keep ACM adequately wetted with amended water from spray equipment before, during, and after removal; do not use compressed air to remove ACM unless it is used with an enclosed capture ventilation system (29 CFR 1926.1101(g)(1)). • Run HEPA-filtered negative air continuously and use HEPA vacuums for pickup — the filters must trap at least 99.97% of 0.3 micrometer particles (para 6-2.k). • Conduct the initial exposure determination and continued personal air monitoring per the air monitoring plan (para 6-7.d(4)(a)-(b)). • Use respirators selected under the written site-specific RPP per 29 CFR 1910.134 (para 5-7.b), upgrading protection factor whenever monitoring or work class requires it. • Bag or wrap removed material while wet as work progresses; do not allow dried debris to accumulate (housekeeping per para 6-7.d(6)). | M |
|  | Heat stress from impermeable coveralls and respirators inside a sealed enclosure | • Establish a work/rest schedule based on monitored heat stress conditions and adjust for the added burden of PPE per the Thermal Stress Management Plan (paras 6-8.m, 6-7.g). • Stage cool potable water and a shaded rest area immediately outside the decon unit; require hydration at each exit. • Schedule gross removal for cooler parts of the shift where feasible and rotate crews through high-exertion tasks. • Implement a buddy system and train workers to recognize heat illness signs in themselves and co-workers; empower any worker to call a break (para 6-8.m(1)). | M |
|  | Slips, trips, and cuts on wet poly, hoses, and sharp lath or mechanical edges | • Maintain clear pathways to airlocks; coil hoses and cords against walls between uses. • Wet-mop and pick up debris continuously rather than allowing piles to build under foot. • Use cut-resistant gloves for scraping and demolition of jagged substrates; keep utility knives retracted when not cutting. | L |
| Waste bag-out, decontamination, and load-out | Contamination spread beyond the regulated area during bag-out and worker exit | • Double-bag or wrap waste in labeled 6-mil bags/poly, HEPA-vacuum and wet-wipe bag exteriors in the airlock before transfer out of containment (paras 6-7.d(6), 6-7.d(10)). • Require full decontamination sequence — HEPA vacuum of suits, suit drop, shower, clean change — for every exit through the decon unit (para 6-7.d(7)). • Clean, inspect, and maintain reusable PPE before reissue; inspect all PPE visually before each use and tag defective equipment out of service (para 5-5.b). • Restrict load-out routes and secure the waste accumulation area per the plan's security procedures (para 6-7.d(11)). | L |
|  | Manual handling injuries from lifting saturated waste bags and drums | • Limit bag fill so wet waste remains a one-person lift or assign two-person lifts and carts for heavier containers. • Stage waste close to the airlock and use drum dollies or carts for transport to the lockable container. • Train crews in lifting technique and rotate bag-out duty during long shifts. | L |
| Final cleaning, visual inspection, and clearance air monitoring | Residual contamination left in place and released to building occupants after teardown | • Perform sequenced final cleaning (HEPA vacuum and wet wipe of all surfaces) followed by a documented visual inspection by the abatement CP before clearance sampling. • Have clearance monitoring conducted by an independent IH or a QP per contract specifications (para 6-7.d(4)(d)); do not remove critical barriers until clearance criteria are met. • Keep negative air units running through final cleaning, settling, and clearance sampling. • Apply lock-down encapsulant to substrate surfaces per the specification before reoccupancy where required by UFGS 02 82 00. | L |
| Containment teardown and demobilization | Fiber release from contaminated poly and equipment during teardown | • Remove containment layers only after written clearance; mist, fold contaminated side inward, and bag all poly as asbestos waste. • HEPA-vacuum and wet-wipe negative air machines, tools, and cords before removal from the regulated area; change or seal filters per the manufacturer's instructions. • Maintain respiratory protection and decon procedures for teardown crews until the last critical barrier and airlock are removed. | L |
|  | Falls during removal of overhead barriers and scaffold breakdown | • Reverse the erection sequence with the same ladder/scaffold controls used during setup; maintain three points of contact. • Keep teardown areas clear of loose poly and debris underfoot before working overhead. | L |
| Waste transport and disposal | Improper transport or disposal of regulated asbestos-containing waste | • Containerize, label, manifest, and dispose of waste at a landfill approved for asbestos per 40 CFR 61 Subpart M, following the plan's waste generation, containerization, transportation, and disposal procedures including recordkeeping (para 6-7.d(10)). • Assign a QP to oversee storage of the waste prior to transportation (para 6-4.b). • Line and enclose transport containers; keep waste adequately wet and secured against release in transit. • Retain waste shipment records and disposal receipts with the project files for the durations required by the NESHAP and the contract. | L |

# Equipment · Training · Inspections

| EQUIPMENT | TRAINING | INSPECTION |
| --- | --- | --- |
| • HEPA-filtered negative air machines with recording manometer (filters rated 99.97% at 0.3 micrometers, para 6-2.k) • HEPA vacuums • Airless sprayers / garden sprayers with amended water for wet methods • 6-mil polyethylene sheeting, spray adhesive, and tape for critical barriers and containment • Three-stage decontamination unit (equipment room, shower, clean room) with heated water • Respirators per the site RPP (half-mask APR minimum; PAPR or supplied air as work class and monitoring require) • Disposable coveralls (with hood/booties), gloves, and eye protection • Labeled 6-mil asbestos waste bags, drums, and a lockable waste container • Personal and area air sampling pumps with calibration equipment (inspected per manufacturer's instructions, para 6-5.a) • GFCI-protected temporary power, cords, and low-voltage containment lighting • Ladders and rolling scaffolds for barrier installation • Plumbed or self-contained emergency eyewash where injurious materials (e.g., encapsulants, amended-water surfactants) are used (provided per para 6-8.e; inspected per para 6-5.b) | • Asbestos training tiered by exposure potential and job responsibilities per applicable OSHA/EPA standards — EPA/AHERA-accredited worker and contractor/supervisor courses with current refreshers for Class I/II work (EM 385-1-1 para 6-3.h(2)) • Respiratory protection training: hazards, selection, donning/doffing, user seal checks, limitations, and maintenance, plus annual fit testing and medical evaluation (paras 5-3.c, 5-7.b; 29 CFR 1910.134) • Medical surveillance for asbestos-exposed workers and respirator users per the Asbestos Management Plan (para 6-7.d(9)) and 29 CFR 1926.1101(m) • Hazard communication training on chemicals in use (amended water surfactants, encapsulants, mastics removers) with SDSs attached to the AHA (paras 6-3.b, 6-6) • AHA review and signature by every involved employee before starting work, and after any update for changed conditions (para 2-6.d(2), 2-6.d(5)) • Thermal stress training for crews working in impermeable PPE: illnesses and injuries, signs and symptoms, prevention, and first aid (paras 6-3.c, 6-7.g) | • Pre-abatement and daily containment inspections: smoke-test barriers/airlocks and verify negative pressure on the recording manometer at the frequency set in the Asbestos Management Plan • Ventilation (negative air) systems evaluated to confirm they function as designed; at least annually for continued service (para 6-5.c) • Air sampling and monitoring equipment inspected and calibrated per the manufacturer's instructions (para 6-5.a) • Personal air monitoring per the air monitoring plan; environmental monitoring to demonstrate no fiber migration outside the regulated area; independent clearance monitoring before teardown (para 6-7.d(4)) • PPE visually inspected before each use; defective equipment tagged out of service and removed; reused PPE cleaned, inspected, and repaired before reissue (para 5-5.b) • Plumbed eyewash units activated and documented weekly; self-contained units visually inspected weekly; annual ANSI/ISEA Z358.1 compliance inspection (para 6-5.b) • GFCI devices and temporary power inspected per EM 385-1-1 Ch 11 requirements • For abatement inside a demolition/renovation DFOW: CP inspections throughout demolition for weakened or deteriorated floors, walls, or loosened material at the frequency in the Demolition Plan (para 17-5) |

# Competent / Qualified Person Designation

Asbestos Competent Person (abatement supervisor) per 29 CFR 1926.1101(o) — EPA/AHERA contractor/supervisor accredited, on site for Class I/II work; designated in writing by name on the AHA (para 2-6.d(4)(g))

Respiratory Protection Program Administrator (RPPA) qualified and trained per para 5-3.c(1) to administer the written site-specific RPP (para 5-7.b)

Independent Industrial Hygienist or Qualified Person for clearance air monitoring per contract specifications (para 6-7.d(4)(d)); an IH performs the hazard evaluation and reviews monitoring results (para 6-4.a)

Qualified Person (QP) overseeing storage of hazardous wastes prior to transportation (para 6-4.b)

Trained and licensed asbestos inspectors for the pre-work survey when the abatement supports demolition/renovation (para 17-8.b)

Designated CP/QP (name / role / proof of competency): ____________________

# Approval Authority

| PRINTED NAME | PRINTED TITLE | DATE | SIGNATURE |
| --- | --- | --- | --- |

Approval Authority is based on overall RAC per EM 385-1-1 (2024), para 1-6 or 2-6, as applicable.

# Employee Documentation Sheet

AHAs must be provided to and reviewed by all involved employees prior to starting the task. Each employee must document their review with a signature (EM 385-1-1 2024, para 2-6.d(2)).

| NAME | OCCUPATION / DESIGNATION (CP/QP) | SIGNATURE | DATE |
| --- | --- | --- | --- |

# References

EM 385-1-1 (2024) 6-3.h(2) — Exposure Potential Training — Asbestos (training and certification tiered by exposure potential per OSHA/EPA standards)

EM 385-1-1 (2024) 6-7.d — Asbestos Management Plan (written plan required when work may disturb ACM; 11 required elements)

EM 385-1-1 (2024) 6-8.f(1) — Asbestos Hazard Control (evaluate all projects for ACM contact; label ACM or post site map; no friable ACM brought onto USACE projects)

EM 385-1-1 (2024) 6-5 — Ch 6 Inspection Requirements (monitoring equipment, eyewash units, ventilation systems)

EM 385-1-1 (2024) 6-6 — Ch 6 AHA Requirements (incorporate SDS information; attach SDS to AHA)

EM 385-1-1 (2024) 5-7.b — Respiratory Protection Program (written site-specific RPP per 29 CFR 1910.134)

EM 385-1-1 (2024) 5-3.c — Respiratory Protection Training/Qualification (RPPA, medical evaluation, fit testing)

EM 385-1-1 (2024) 5-5.b — PPE Inspection Requirements (visual inspection before each use)

EM 385-1-1 (2024) 17-8.b — Lead, Asbestos, and ORM Surveys before demolition/renovation (trained and licensed inspectors per Ch 6)

EM 385-1-1 (2024) 17-8.d(3) — Abatement of lead, asbestos, and ORM completed before structural demolition begins

EM 385-1-1 (2024) 2-6 — Contractor AHA requirements (hierarchy of controls, residual RAC, CP/QP designation by name)

29 CFR 1926.1101 — Asbestos (Construction, Subpart Z) — work classes, PEL 0.1 f/cc TWA, competent person, regulated areas

29 CFR 1910.134 — Respiratory Protection — medical evaluation, fit testing, written program

40 CFR 61 Subpart M — National Emission Standard for Asbestos (NESHAP) — notification, emission controls, waste disposal

UFGS 02 82 00 — Asbestos Remediation — typical contract specification for federal abatement work